Arduino is a great starting point for electronics, and with a motor shield it can also be a nice tidy platform for robotics and mechatronics. Here is a design for a full-featured motor shield that will be able to power many simple to medium-complexity projects.

- 2 connections for 5V 'hobby' servos connected to the Arduino's high-resolution dedicated timer - no jitter!

- Up to 4 bi-directional DC motors with individual 8-bit speed selection (so, about 0.5% resolution)

- Up to 2 stepper motors (unipolar or bipolar) with single coil, double coil, interleaved or micro-stepping.

- 4 H-Bridges: L293D chipset provides 0.6A per bridge (1.2A peak) with thermal shutdown protection, 4.5V to12V

- Pull down resistors keep motors disabled during power-up

- Big terminal block connectors to easily hook up wires (10-22AWG) and power

- Arduino reset button brought up top

- 2-pin terminal block to connect external power, for seperate logic/motor supplies

- Tested compatible with Mega, Diecimila, & Duemilanove

- Accessories program in Arduino-022 version, if you need to run on other versions, you may need to modify the herader file.